Команды настройки сети

Вариант №1: GRE, OSPF, DHCP, VLAN, STP, EtherChannel, SSH, Port Security

Схема сети из задания

Примечание: номера интерфейсов приняты по типовой схеме Packet Tracer. Если в твоей схеме используются другие порты, замени только имена интерфейсов, не меняя логику команд.

# 1. Принятые подключения

| Линк | Интерфейсы |
| --- | --- |
| ISP — R1 | ISP G0/0 ↔ R1 G0/0 |
| ISP — R2 | ISP G0/1 ↔ R2 G0/0 |
| R1 — SW1 | R1 G0/1 ↔ SW1 G0/1 |
| SW1 — SW2 | SW1 F0/1 ↔ SW2 F0/1 |
| SW1 — SW3 | SW1 F0/2 ↔ SW3 F0/1 |
| SW2 — SW3 | SW2 F0/2 ↔ SW3 F0/2 |
| R2 — SW4 | R2 G0/1 ↔ SW4 G0/1 |
| SW4 — SW5 EtherChannel | SW4 F0/23-24 ↔ SW5 F0/23-24 |

# 2. IP-адресация

| Устройство | Интерфейс | IP |
| --- | --- | --- |
| ISP | G0/0 | 10.0.0.1/30 |
| ISP | G0/1 | 10.0.0.5/30 |
| R1 | G0/0 | 10.0.0.2/30 |
| R1 | G0/1.10 | 192.168.10.1/26 |
| R1 | G0/1.20 | 192.168.10.65/27 |
| R1 | G0/1.30 | 192.168.10.97/27 |
| R1 | Tunnel0 | 192.168.255.1/30 |
| R2 | G0/0 | 10.0.0.6/30 |
| R2 | G0/1 | 172.16.20.1/24 |
| R2 | Tunnel0 | 192.168.255.2/30 |
| SW1 | VLAN 10 | 192.168.10.2/26 |
| SW2 | VLAN 10 | 192.168.10.3/26 |
| SW3 | VLAN 10 | 192.168.10.4/26 |
| SW4 | VLAN 1 | 172.16.20.2/24 |
| SW5 | VLAN 1 | 172.16.20.3/24 |

# 3. VLAN Left

| VLAN | Подсеть | Gateway |
| --- | --- | --- |
| VLAN 10 | 192.168.10.0/26 | 192.168.10.1 |
| VLAN 20 | 192.168.10.64/27 | 192.168.10.65 |
| VLAN 30 | 192.168.10.96/27 | 192.168.10.97 |

# 4. Команды настройки устройств

## ISP

enable

conf t

hostname ISP

enable secret securepass1

service password-encryption

line console 0

password console123

login

logging synchronous

exit

ip domain-name example.com

username admin secret sshpassword1

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

interface g0/0

description TO-R1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

interface g0/1

description TO-R2

ip address 10.0.0.5 255.255.255.252

no shutdown

exit

end

wr

## R1

enable

conf t

hostname R1

enable secret securepass1

service password-encryption

line console 0

password console123

login

logging synchronous

exit

ip domain-name example.com

username admin secret sshpassword1

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

interface g0/0

description TO-ISP

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

interface g0/1

description TO-SW1-TRUNK

no ip address

no shutdown

exit

interface g0/1.10

description VLAN10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.192

exit

interface g0/1.20

description VLAN20

encapsulation dot1Q 20

ip address 192.168.10.65 255.255.255.224

exit

interface g0/1.30

description VLAN30

encapsulation dot1Q 30

ip address 192.168.10.97 255.255.255.224

exit

ip route 10.0.0.4 255.255.255.252 10.0.0.1

interface Tunnel0

description GRE-TO-R2

ip address 192.168.255.1 255.255.255.252

tunnel source g0/0

tunnel destination 10.0.0.6

no shutdown

exit

ip dhcp excluded-address 192.168.10.1 192.168.10.10

ip dhcp excluded-address 192.168.10.65 192.168.10.70

ip dhcp excluded-address 192.168.10.97 192.168.10.100

ip dhcp pool VLAN10

network 192.168.10.0 255.255.255.192

default-router 192.168.10.1

dns-server 8.8.8.8

exit

ip dhcp pool VLAN20

network 192.168.10.64 255.255.255.224

default-router 192.168.10.65

dns-server 8.8.8.8

exit

ip dhcp pool VLAN30

network 192.168.10.96 255.255.255.224

default-router 192.168.10.97

dns-server 8.8.8.8

exit

router ospf 1

router-id 1.1.1.1

passive-interface default

no passive-interface Tunnel0

network 192.168.255.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.63 area 0

network 192.168.10.64 0.0.0.31 area 0

network 192.168.10.96 0.0.0.31 area 0

exit

end

wr

## R2

enable

conf t

hostname R2

enable secret securepass1

service password-encryption

line console 0

password console123

login

logging synchronous

exit

ip domain-name example.com

username admin secret sshpassword1

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

interface g0/0

description TO-ISP

ip address 10.0.0.6 255.255.255.252

no shutdown

exit

interface g0/1

description TO-SW4

ip address 172.16.20.1 255.255.255.0

no shutdown

exit

ip route 10.0.0.0 255.255.255.252 10.0.0.5

interface Tunnel0

description GRE-TO-R1

ip address 192.168.255.2 255.255.255.252

tunnel source g0/0

tunnel destination 10.0.0.2

no shutdown

exit

ip dhcp excluded-address 172.16.20.1 172.16.20.20

ip dhcp pool RIGHT

network 172.16.20.0 255.255.255.0

default-router 172.16.20.1

dns-server 8.8.8.8

exit

router ospf 1

router-id 2.2.2.2

passive-interface default

no passive-interface Tunnel0

network 192.168.255.0 0.0.0.3 area 0

network 172.16.20.0 0.0.0.255 area 0

exit

end

wr

## SW1

enable

conf t

hostname SW1

enable secret securepass1

service password-encryption

line console 0

password console123

login

logging synchronous

exit

ip domain-name example.com

username admin secret sshpassword1

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

vlan 10

name VLAN10

exit

vlan 20

name VLAN20

exit

vlan 30

name VLAN30

exit

interface vlan 10

ip address 192.168.10.2 255.255.255.192

no shutdown

exit

ip default-gateway 192.168.10.1

interface g0/1

description TO-R1

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

exit

interface f0/1

description TO-SW2

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

exit

interface f0/2

description TO-SW3

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

exit

spanning-tree mode pvst

spanning-tree vlan 10,20,30 priority 4096

end

wr

## SW2

enable

conf t

hostname SW2

enable secret securepass1

service password-encryption

line console 0

password console123

login

logging synchronous

exit

ip domain-name example.com

username admin secret sshpassword1

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

vlan 10

name VLAN10

exit

vlan 20

name VLAN20

exit

vlan 30

name VLAN30

exit

interface vlan 10

ip address 192.168.10.3 255.255.255.192

no shutdown

exit

ip default-gateway 192.168.10.1

interface f0/1

description TO-SW1

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

exit

interface f0/2

description TO-SW3

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

exit

interface range f0/10-19

description VLAN10-ACCESS

switchport mode access

switchport access vlan 10

switchport port-security

switchport port-security mac-address sticky

switchport port-security violation restrict

spanning-tree portfast

spanning-tree bpduguard enable

no shutdown

exit

interface range f0/20-29

description VLAN20-ACCESS

switchport mode access

switchport access vlan 20

switchport port-security

switchport port-security mac-address sticky

switchport port-security violation restrict

spanning-tree portfast

spanning-tree bpduguard enable

no shutdown

exit

spanning-tree mode pvst

spanning-tree vlan 10,20,30 priority 8192

end

wr

## SW3

enable

conf t

hostname SW3

enable secret securepass1

service password-encryption

line console 0

password console123

login

logging synchronous

exit

ip domain-name example.com

username admin secret sshpassword1

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

vlan 10

name VLAN10

exit

vlan 20

name VLAN20

exit

vlan 30

name VLAN30

exit

interface vlan 10

ip address 192.168.10.4 255.255.255.192

no shutdown

exit

ip default-gateway 192.168.10.1

interface f0/1

description TO-SW1

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

exit

interface f0/2

description TO-SW2

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

exit

interface range f0/10-19

description VLAN30-ACCESS

switchport mode access

switchport access vlan 30

switchport port-security

switchport port-security mac-address sticky

switchport port-security violation restrict

spanning-tree portfast

spanning-tree bpduguard enable

no shutdown

exit

spanning-tree mode pvst

spanning-tree vlan 10,20,30 priority 12288

end

wr

## SW4

enable

conf t

hostname SW4

enable secret securepass1

service password-encryption

line console 0

password console123

login

logging synchronous

exit

ip domain-name example.com

username admin secret sshpassword1

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

interface vlan 1

ip address 172.16.20.2 255.255.255.0

no shutdown

exit

ip default-gateway 172.16.20.1

interface g0/1

description TO-R2

switchport mode access

switchport access vlan 1

no shutdown

exit

interface range f0/23-24

description LACP-TO-SW5

switchport mode trunk

channel-protocol lacp

channel-group 1 mode active

no shutdown

exit

interface port-channel 1

description LACP-TO-SW5

switchport mode trunk

no shutdown

exit

interface f0/10

description TO-PC3

switchport mode access

switchport access vlan 1

switchport port-security

switchport port-security mac-address sticky

switchport port-security violation restrict

spanning-tree portfast

spanning-tree bpduguard enable

no shutdown

exit

end

wr

## SW5

enable

conf t

hostname SW5

enable secret securepass1

service password-encryption

line console 0

password console123

login

logging synchronous

exit

ip domain-name example.com

username admin secret sshpassword1

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

interface vlan 1

ip address 172.16.20.3 255.255.255.0

no shutdown

exit

ip default-gateway 172.16.20.1

interface range f0/23-24

description LACP-TO-SW4

switchport mode trunk

channel-protocol lacp

channel-group 1 mode active

no shutdown

exit

interface port-channel 1

description LACP-TO-SW4

switchport mode trunk

no shutdown

exit

interface f0/10

description TO-PC4

switchport mode access

switchport access vlan 1

switchport port-security

switchport port-security mac-address sticky

switchport port-security violation restrict

spanning-tree portfast

spanning-tree bpduguard enable

no shutdown

exit

end

wr

# 5. Настройка ПК

На всех ПК включить DHCP.

| ПК | Коммутатор/порт | VLAN | DHCP-сеть |
| --- | --- | --- | --- |
| PC0 | SW2 F0/10 | 10 | 192.168.10.0/26 |
| PC5 | SW2 F0/11 | 10 | 192.168.10.0/26 |
| PC2 | SW2 F0/20 | 20 | 192.168.10.64/27 |
| PC1 | SW3 F0/10 | 30 | 192.168.10.96/27 |
| PC3 | SW4 F0/10 | 1 | 172.16.20.0/24 |
| PC4 | SW5 F0/10 | 1 | 172.16.20.0/24 |

# 6. Проверка

show ip interface brief

show interfaces tunnel 0

show ip ospf neighbor

show ip route ospf

show spanning-tree vlan 10

show spanning-tree vlan 20

show spanning-tree vlan 30

show etherchannel summary

show port-security

show port-security interface f0/10

show ip dhcp binding

# 7. Проверка связности

ping 192.168.255.2

ping 172.16.20.1

ping 192.168.10.1

ping 192.168.10.65

ping 192.168.10.97

# 8. Если WAN-интерфейсы Serial

Если в Packet Tracer у роутеров WAN-интерфейсы Serial, вместо G0/0/G0/1 используй свои S0/0/0 и S0/0/1. На DCE-стороне добавь:

clock rate 64000